Настройка маршрутизатора RT1:

[HPE954](config)#hostname RT1

[RT1](config)#ip routing

[RT1](config)#ipv6 unicast-routing

[RT1](config)#interface GigabitEthernet0/0

[RT1](config-if)#ip address dhcp

[RT1](config-if)#no shutdown

[RT1](config)#interface GigabitEthernet0/1.10

[RT1](config-if)#ip address 202.118.74.1 255.255.255.224

[RT1](config-if)#encapsulation dot1q 10

[RT1](config)#interface GigabitEthernet0/1.20

[RT1](config-if)#ip address 202.118.74.193 255.255.255.192

[RT1](config-if)#ipv6 address fd00:0:0:20::1/64

[RT1](config-if)#encapsulation dot1q 20

[RT1](config)#interface GigabitEthernet0/1.30

[RT1](config-if)#ip address 202.118.74.129 255.255.255.192

[RT1](config-if)#ipv6 address fd00:0:0:30::1/64

[RT1](config-if)#encapsulation dot1q 30

[RT1](config)#interface GigabitEthernet0/1.40

[RT1](config-if)#ip address 202.118.74.65 255.255.255.192

[RT1](config-if)#ipv6 address fd00:0:0:40::1/64

[RT1](config-if)#encapsulation dot1q 40

[RT1](config)#interface GigabitEthernet0/1.50

[RT1](config-if)#ip address 202.118.74.33 255.255.255.240

[RT1](config-if)#ipv6 address fd00:0:0:50::1/64

[RT1](config-if)#encapsulation dot1q 50

[RT1](config)#interface GigabitEthernet0/1.60

[RT1](config-if)#ip address 202.118.74.49 255.255.255.240

[RT1](config-if)#ipv6 address fd00:0:0:60::1/64

[RT1](config-if)#encapsulation dot1q 60

[RT1](config)#interface GigabitEthernet0/1

[RT1](config)#mvrp enable

[RT1](config-if)#no shutdown

[RT1](config)#ip dhcp pool Lab

[RT1](dhcp-config)#network 202.118.74.0 255.255.255.192

[RT1](dhcp-config)#range 202.118.74.2 202.118.74.68

[RT1](dhcp-config)#default-router 202.118.74.1

[RT1](config)#ipv6 dhcp pool LabIPv6

[RT1](ipv6-dhcp-config)#prefix fd00:0:0:20::/64

[RT1](ipv6-dhcp-config)#address fd00:0:0:20::2

…

[RT1](ipv6-dhcp-config)#address fd00:0:0:20::44

[RT1](ipv6-dhcp-config)#ipv6 default-router fd00:0:0:20::1

[RT1](config)#ip route 202.118.74.128 255.255.255.192 202.118.74.130

[RT1](config)#ip dhcp pool Employee

[RT1](dhcp-config)#network 202.118.74.64 255.255.255.192

[RT1](dhcp-config)#range 202.118.74.66 202.118.74.86

[RT1](dhcp-config)#default-router 202.118.74.65

[RT1](config)#ipv6 dhcp pool Employee IPv6

[RT1](ipv6-dhcp-config)#prefix fd00:0:0:40::/64

[RT1](ipv6-dhcp-config)#address fd00:0:0:40::2

…

[RT1](ipv6-dhcp-config)#address fd00:0:0:40::16

[RT1](ipv6-dhcp-config)#ipv6 default-router fd00:0:0:40::1

[RT1](config)#ip access-list ALLOW_SERVER

[RT1](config-list)#permit ip 202.118.74.0 255.255.255.0 any

[RT1](config-list)#deny ip any any

[RT1](config)#ip access-list ALLOW_ADMIN

[RT1](config-list)#permit ip 202.118.74.0 255.255.255.224 any

[RT1](config-list)#deny ip any any

[RT1](config)#interface GigabitEthernet0/1.10

[RT1](config-if)#ip access-group ALLOW_ADMIN in

[RT1](config)#interface GigabitEthernet0/1.50

[RT1](config-if)#ip access-group ALLOW_SERVER in

[RT1](config)#ip route 202.118.74.128 255.255.255.192 202.118.74.130

Настройка прокси-сервера SRV2:

#!/bin/bash

EXPECTED_HOSTNAME="SRV2"

CURRENT_HOSTNAME=$(hostname)

if [[ "$CURRENT_HOSTNAME" != "$EXPECTED_HOSTNAME" ]]; then

echo "Этот скрипт должен быть запущен только на сервере $EXPECTED_HOSTNAME."

exit 1

fi

INTERFACE="GigabitEthernet0"

LOGFILE="/path/to/logfile.pcap"

EXCLUDED_SUBNETS=("202.118.74.0/24")

FILTER=""

for subnet in "${EXCLUDED_SUBNETS[@]}"; do

FILTER="$FILTER and not \(src net $subnet and dst net $subnet\) "

done

# Убираем лишний пробел в конце фильтра

FILTER=${FILTER#and }

echo "Начинаю логирование трафика на $INTERFACE, исключая подсети: ${EXCLUDED_SUBNETS[*]}"

sudo tcpdump -i "$INTERFACE" "$FILTER" -w "$LOGFILE"