Конфигурация коммутатора SW1:

enable

configure

hostname SW1

service password-encryption

enable secret 0 sh@sji!4jFpo

username admin password 0 sh@sji!4jFpo privilege admin

ip ssh server

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

vlan 10

name ADMIN

exit

vlan 20

name EMP_WIRED

exit

vlan 30

name EMP_WIFI

exit

vlan 40

name GUEST

exit

vlan 50

name SERVERS

exit

interface vlan 10

ip address 192.168.10.2 255.255.255.240

exit

ip management-vlan 10

power inline consumption 160

power profile "AP"

supply enable

priority high

consumption class4

exit

interface 1

vlan_trunk

switchport general allowed vlan 10,20,30,40,50 tagged

exit

interface 15

vlan_trunk

switchport general allowed vlan 10,20,30,40,50 tagged

exit

interface 2

switchport general allowed vlan 10 untagged

switchport general allowed vlan 30 tagged

switchport pvid 10

power inline supply enable

power inline profile "AP"

exit

interface 3

switchport general allowed vlan 10 untagged

switchport general allowed vlan 30 tagged

switchport pvid 10

power inline supply enable

power inline profile "AP"

exit

interface 4

switchport general allowed vlan 50 untagged

switchport pvid 50

exit

interface 5

switchport general allowed vlan 50 untagged

switchport pvid 50

exit

interface 6

switchport general allowed vlan 10 untagged

switchport pvid 10

exit

interface range 7-13

switchport general allowed vlan 20 untagged

switchport pvid 20

exit

interface 14

switchport general allowed vlan 50 untagged

switchport pvid 50

exit

interface 16

shutdown

exit

end