/system reset-configuration

/interface vlan

add name=VLAN100 vlan-id=100 interface=bridge

add name=VLAN200 vlan-id=200 interface=bridge

add name=VLAN300 vlan-id=300 interface=bridge

/ip address

add address=109.44.129.66/27 interface=VLAN100

add address=109.44.129.1/26 interface=VLAN200

add address=109.44.128.1/24 interface=VLAN300

/interface bridge

add name=bridge

/interface bridge port

add interface=ether0/1/0 bridge=bridge

add interface=ether0/1/1 bridge=bridge

/ip route

add dst-address=8.8.4.4 gateway=127.0.0.1 scope=30 target-scope=30 comment="isp1route" disabled=yes

add dst-address=0.0.0.0/0 gateway=8.8.4.4%sfp1 distance=1 check-gateway=ping comment="Основной маршрут"

/ip route

add dst-address=8.8.8.8 gateway=127.0.0.1 scope=30 target-scope=30 comment="isp2route" disabled=yes

add dst-address=0.0.0.0/0 gateway=8.8.8.8%sfp2 distance=2 check-gateway=ping comment="Резервный маршрут"

/system script

add name=UpdateISP1Route source="

:local gw [/ip dhcp-client get [find interface=sfp1] gateway-address];

/ip route set [find comment='isp1route'] gateway=\$gw disabled=no;

:log info \"Обновлен основной маршрут: \$gw\";"

/system script

add name=UpdateISP2Route source="

:local gw [/ip dhcp-client get [find interface=sfp2] gateway-address];

/ip route set [find comment='isp2route'] gateway=\$gw disabled=no;

:log info \"Обновлен резервный маршрут: \$gw\";"

/ip dhcp-client

add interface=sfp1 default-route-distance=255 comment="route1isp"

add interface=sfp2 default-route-distance=255 comment="route2isp"

set [find interface=sfp1] script="UpdateISP1Route"

set [find interface=sfp2] script="UpdateISP2Route"

/ip firewall nat

add chain=srcnat action=masquerade out-interface=sfp1 src-address=109.44.129.64/27

add chain=srcnat action=masquerade out-interface=sfp1 src-address=109.44.129.0/26

add chain=srcnat action=masquerade out-interface=sfp1 src-address=109.44.128.0/24

add chain=srcnat action=masquerade out-interface=sfp2 src-address=109.44.129.64/27

add chain=srcnat action=masquerade out-interface=sfp2 src-address=109.44.129.0/26

add chain=srcnat action=masquerade out-interface=sfp2 src-address=109.44.128.0/24

ip dhcp-server

add name=dhcp_vlan300 interface=vlan300 address-pool=dhcp_pool_vlan300 lease-time=1h

/ip dhcp-server network

add address=109.44.128.0/24 gateway=109.44.128.1

/caps-man security

add name=vlan300_security authentication-types=wpa2-psk encryption=aes-ccm passphrase=racer1626

/caps-man configuration

add name=vlan300_config ssid=”Web-design deps” security=vlan300_security datapath.vlan-id=300 datapath.vlan-mode=use-tag

/caps-man provisioning

add action=create-enabled master-configuration=vlan300_config

/ip firewall filter add chain=forward src-address=109.44.128.0/24 dst-address=109.44.129.2 action=drop

/ip firewall filter add chain=forward src-address=109.44.129.0/26 dst-address=109.44.129.2 action=accept

/ip pool add name=dhcp_pool_vlan200 ranges=109.44.129.3-109.44.129.47

/ip dhcp-server

add name=dhcp_vlan200 interface=vlan200 address-pool=dhcp_pool_vlan200 lease-time=24h

/ip dhcp-server network

add address=109.44.129.0/26 gateway=109.44.129.1

/ipv6 address

add address=2001:67c:18a8:1::1/64 interface=vlan100

add address=2001:67c:18a8:2::1/64 interface=vlan200

add address=2001:67c:18a8:3::1/64 interface=vlan300

add address=2001:67c:18a8:4::2/64 interface=sfp1

add address=2001:67c:18a8:5::2/64 interface=sfp2

/ipv6 route add dst-address=::/0 gateway=2001:67c:18a8:4::1%sfp1 distance=1

/ipv6 route add dst-address=::/0 gateway=2001:67c:18a8:5::1%sfp2 distance=2

ipv6 dhcp−server> add name=dhcp_server200v6 interface=vlan300 address−pool=dhcp_pool300v6, exclude-pool= dhcp_pool300v6_exclude

ipv6 pool> add name=dhcp_pool300v6 prefix=2001:67c:18a8:2::/64

ipv6 dhcp−server> add name=dhcp_wireless300v6 interface=vlan300 address−pool=dhcp_pool300v6, exclude-pool= dhcp_pool300v6_exclude

ipv6 pool> add name=dhcp_pool300v6 prefix=2001:67c:18a8:3::/64

Коммутатор SW1.1

/system reset-configuration

/interface bridge

add name=bridge1 vlan-filtering=yes

/interface bridge port

add interface=ether1 bridge=bridge1

add interface=ether2 bridge=bridge1 pvid=300

add interface=ether3 bridge=bridge1 pvid=300

add interface=ether4 bridge=bridge1 pvid=300

add interface=ether5 bridge=bridge1 pvid=300

add interface=ether6 bridge=bridge1 pvid=300

add interface=ether7 bridge=bridge1 pvid=300

add interface=ether8 bridge=bridge1 pvid=200

add interface=ether9 bridge=bridge1 pvid=200

add interface=ether10 bridge=bridge1 pvid=200

add interface=ether11 bridge=bridge1 pvid=200

add interface=ether12 bridge=bridge1 pvid=200

add interface=ether13 bridge=bridge1 pvid=200

add interface=ether14 bridge=bridge1 pvid=200

add interface=ether15 bridge=bridge1 pvid=200

add interface=ether16 bridge=bridge1

/interface bridge vlan

add bridge=bridge1 tagged=ether1,ether16 vlan-ids=100

add bridge=bridge1 tagged=ether1,ether16 untagged=ether8,ether9,ether10,ether11,ether12,ether13,ether14, ether15 vlan-ids=200

add bridge=bridge1 tagged=ether1,ether16 untagged=ether2,ether3,ether4,ether5,ether6,ether7 vlan-ids=300

/interface ethernet switch port

set ether2 default-vlan-id=300 vlan-header=always-strip

set ether3 default-vlan-id=300 vlan-header=always-strip

set ether4 default-vlan-id=300 vlan-header=always-strip

set ether5 default-vlan-id=300 vlan-header=always-strip

set ether6 default-vlan-id=300 vlan-header=always-strip

set ether7 default-vlan-id=300 vlan-header=always-strip

set ether8 default-vlan-id=200 vlan-header=always-strip

set ether9 default-vlan-id=200 vlan-header=always-strip

set ether10 default-vlan-id=200 vlan-header=always-strip

set ether11 default-vlan-id=200 vlan-header=always-strip

set ether12 default-vlan-id=200 vlan-header=always-strip

set ether13 default-vlan-id=200 vlan-header=always-strip

set ether14 default-vlan-id=200 vlan-header=always-strip

set ether15 default-vlan-id=200 vlan-header=always-strip

set ether16 vlan-header=add-if-missing

set ether17 disabled=yes

set ether18 disabled=yes

set ether19 disabled=yes

set ether20 disabled=yes

set ether21 disabled=yes

set ether22 disabled=yes

set ether23 disabled=yes

set ether24 disabled=yes

/interface bridge filter

add action=drop chain=forward in-interface=ether14 src-mac-address=!00:1A:2B:3C:4D:5E

add action=drop chain=forward in-interface=ether8 src-mac-address=!00:1B:2C:3D:4E:5F

add action=drop chain=forward in-interface=ether9 src-mac-address=!00:1C:2D:3E:4F:5A

add action=drop chain=forward in-interface=ether10 src-mac-address=!00:1D:2E:3F:4A:5B

add action=drop chain=forward in-interface=ether11 src-mac-address=!00:1E:2F:3A:4B:5C

add action=drop chain=forward in-interface=ether12 src-mac-address=!00:1F:2A:3B:4C:5D

add action=drop chain=forward in-interface=ether13 src-mac-address=!00:2A:3B:4C:5D:6E

add action=drop chain=forward in-interface=ether15 src-mac-address=!00:2B:3C:4D:5E:6F

/interface vlan

add name=VLAN100 vlan-id=100 interface=bridge1

/ip address

add address=109.44.129.77/27 interface=VLAN100

/ipv6 address

add address=2001:67c:18a8:1::13 interface=VLAN100

Коммутатор SW1.2

/system reset-configuration

/interface bridge

add name=bridge1 vlan-filtering=yes

/interface bridge port

add interface=ether0 bridge=bridge1

add interface=ether1 bridge=bridge1 pvid=200

add interface=ether2 bridge=bridge1 pvid=200

add interface=ether3 bridge=bridge1 pvid=200

add interface=ether4 bridge=bridge1 pvid=200

add interface=ether5 bridge=bridge1 pvid=200

add interface=ether6 bridge=bridge1 pvid=200

add interface=ether7 bridge=bridge1 pvid=200

add interface=ether8 bridge=bridge1 pvid=200

add interface=ether9 bridge=bridge1 pvid=200

add interface=ether10 bridge=bridge1 pvid=200

add interface=ether11 bridge=bridge1 pvid=200

add interface=ether12 bridge=bridge1 pvid=200

add interface=ether13 bridge=bridge1 pvid=200

add interface=ether14 bridge=bridge1 pvid=200

add interface=ether15 bridge=bridge1 pvid=200

add interface=ether16 bridge=bridge1 pvid=200

/interface bridge vlan

add bridge=bridge1 tagged=ether0 vlan-ids=100

add bridge=bridge1 tagged=ether0 untagged=ether1,ether2,ether3,ether4,ether5,ether6,ether7,ether8,ether9,ether10,ether11,ether12,ether13,ether14,ether15,ether16 vlan-ids=200

/interface ethernet switch port

set ether1 default-vlan-id=200 vlan-header=always-strip

set ether2 default-vlan-id=200 vlan-header=always-strip

set ether3 default-vlan-id=200 vlan-header=always-strip

set ether4 default-vlan-id=200 vlan-header=always-strip

set ether5 default-vlan-id=200 vlan-header=always-strip

set ether6 default-vlan-id=200 vlan-header=always-strip

set ether7 default-vlan-id=200 vlan-header=always-strip

set ether8 default-vlan-id=200 vlan-header=always-strip

set ether9 default-vlan-id=200 vlan-header=always-strip

set ether10 default-vlan-id=200 vlan-header=always-strip

set ether11 default-vlan-id=200 vlan-header=always-strip

set ether12 default-vlan-id=200 vlan-header=always-strip

set ether13 default-vlan-id=200 vlan-header=always-strip

set ether14 default-vlan-id=200 vlan-header=always-strip

set ether15 default-vlan-id=200 vlan-header=always-strip

set ether16 default-vlan-id=200 vlan-header=always-strip

set ether0 vlan-header=add-if-missing

/interface ethernet

set ether17 disabled=yes

set ether18 disabled=yes

set ether19 disabled=yes

set ether20 disabled=yes

set ether21 disabled=yes

set ether22 disabled=yes

set ether23 disabled=yes

set ether24 disabled=yes

/interface bridge filter

add action=drop chain=forward in-interface=ether1 src-mac-address=!00:2C:3D:4E:5F:6A

add action=drop chain=forward in-interface=ether2 src-mac-address=!00:2D:3E:4F:5A:6B

add action=drop chain=forward in-interface=ether3 src-mac-address=!00:2E:3F:4A:5B:6C

add action=drop chain=forward in-interface=ether4 src-mac-address=!00:2F:3A:4B:5C:6D

add action=drop chain=forward in-interface=ether5 src-mac-address=!00:3A:4B:5C:6D:7E

add action=drop chain=forward in-interface=ether6 src-mac-address=!00:3B:4C:5D:6E:7F

add action=drop chain=forward in-interface=ether7 src-mac-address=!00:3C:4D:5E:6F:7A

add action=drop chain=forward in-interface=ether8 src-mac-address=!00:3D:4E:5F:6A:7B

add action=drop chain=forward in-interface=ether9 src-mac-address=!00:3E:4F:5A:6B:7C

add action=drop chain=forward in-interface=ether10 src-mac-address=!00:3F:4A:5B:6C:7D

add action=drop chain=forward in-interface=ether11 src-mac-address=!00:4A:5B:6C:7D:8E

add action=drop chain=forward in-interface=ether12 src-mac-address=!00:4B:5C:6D:7E:8F

add action=drop chain=forward in-interface=ether13 src-mac-address=!00:4C:5D:6E:7F:8A

add action=drop chain=forward in-interface=ether14 src-mac-address=!00:4D:5E:6F:7A:8B

add action=drop chain=forward in-interface=ether15 src-mac-address=!00:4E:5F:6A:7B:8C

add action=drop chain=forward in-interface=ether16 src-mac-address=! 00:5A:6B:7C:8D:9E

/interface vlan

add name=VLAN100 vlan-id=100 interface=bridge1

/ip address

add address=109.44.129.78/27 interface=VLAN100

/ipv6 address

add address=2001:67c:18a8:1::14 interface=VLAN100

Коммутатор SW2.1

/system reset-configuration

/interface bridge

add name=bridge1 vlan-filtering=yes

/interface bridge port

add interface=ether0 bridge=bridge1

add interface=ether1 bridge=bridge1 pvid=300

add interface=ether2 bridge=bridge1 pvid=300

add interface=ether3 bridge=bridge1 pvid=300

add interface=ether4 bridge=bridge1 pvid=300

add interface=ether5 bridge=bridge1 pvid=300

add interface=ether6 bridge=bridge1 pvid=200

add interface=ether7 bridge=bridge1 pvid=200

add interface=ether8 bridge=bridge1 pvid=200

add interface=ether9 bridge=bridge1 pvid=200

add interface=ether10 bridge=bridge1 pvid=200

add interface=ether11 bridge=bridge1

/interface bridge vlan

add bridge=bridge1 tagged=ether0,ether11 vlan-ids=100

add bridge=bridge1 tagged=ether0, ether11 untagged=ether1,ether2,ether3,ether4,ether5 vlan-ids=300

add bridge=bridge1 tagged=ether0, ether11 untagged=ether6,ether7,ether8,ether9,ether10 vlan-ids=200

/interface ethernet switch port

set ether1 default-vlan-id=300 vlan-header=always-strip

set ether2 default-vlan-id=300 vlan-header=always-strip

set ether3 default-vlan-id=300 vlan-header=always-strip

set ether4 default-vlan-id=300 vlan-header=always-strip

set ether5 default-vlan-id=300 vlan-header=always-strip

set ether6 default-vlan-id=200 vlan-header=always-strip

set ether7 default-vlan-id=200 vlan-header=always-strip

set ether8 default-vlan-id=200 vlan-header=always-strip

set ether9 default-vlan-id=200 vlan-header=always-strip

set ether10 default-vlan-id=200 vlan-header=always-strip

set ether0 vlan-header=add-if-missing

/interface ethernet

set ether12 disabled=yes

set ether13 disabled=yes

set ether14 disabled=yes

set ether15 disabled=yes

set ether16 disabled=yes

set ether17 disabled=yes

set ether18 disabled=yes

set ether19 disabled=yes

set ether20 disabled=yes

set ether21 disabled=yes

set ether22 disabled=yes

set ether23 disabled=yes

set ether24 disabled=yes

/interface bridge filter

add action=drop chain=forward in-interface=ether6 src-mac-address=!00:5C:6D:7E:8F:9A

add action=drop chain=forward in-interface=ether7 src-mac-address=!00:5D:6E:7F:8A:9B

add action=drop chain=forward in-interface=ether8 src-mac-address=!00:5E:6F:7A:8B:9C

add action=drop chain=forward in-interface=ether9 src-mac-address=!00:5F:6A:7B:8C:9D

add action=drop chain=forward in-interface=ether10 src-mac-address=!00:6A:7B:8C:9D:AE

/interface vlan

add name=VLAN100 vlan-id=100 interface=bridge1

/ip address

add address=109.44.129.79/27 interface=VLAN100

/ipv6 address

add address=2001:67c:18a8:1::15 interface=VLAN100

Коммутатор SW2.2

/system reset-configuration

/interface bridge

add name=bridge1 vlan-filtering=yes

/interface bridge port

add interface=ether0 bridge=bridge1

add interface=ether1 bridge=bridge1 pvid=200

add interface=ether2 bridge=bridge1 pvid=200

add interface=ether3 bridge=bridge1 pvid=200

add interface=ether4 bridge=bridge1 pvid=200

add interface=ether5 bridge=bridge1 pvid=200

add interface=ether6 bridge=bridge1 pvid=200

add interface=ether7 bridge=bridge1 pvid=200

add interface=ether8 bridge=bridge1 pvid=200

add interface=ether9 bridge=bridge1 pvid=200

add interface=ether10 bridge=bridge1 pvid=200

add interface=ether11 bridge=bridge1 pvid=200

add interface=ether12 bridge=bridge1 pvid=200

add interface=ether13 bridge=bridge1 pvid=200

add interface=ether14 bridge=bridge1 pvid=200

add interface=ether15 bridge=bridge1 pvid=200

add interface=ether16 bridge=bridge1 pvid=200

/interface bridge vlan

add bridge=bridge1 tagged=ether0 vlan-ids=100

add bridge=bridge1 tagged=ether0 untagged=ether1,ether2,ether3,ether4,ether5,ether6,ether7,ether8,ether9,ether10,ether11,ether12,ether13,ether14,ether15,ether16 vlan-ids=200

/interface ethernet switch port

set ether1 default-vlan-id=200 vlan-header=always-strip

set ether2 default-vlan-id=200 vlan-header=always-strip

set ether3 default-vlan-id=200 vlan-header=always-strip

set ether4 default-vlan-id=200 vlan-header=always-strip

set ether5 default-vlan-id=200 vlan-header=always-strip

set ether6 default-vlan-id=200 vlan-header=always-strip

set ether7 default-vlan-id=200 vlan-header=always-strip

set ether8 default-vlan-id=200 vlan-header=always-strip

set ether9 default-vlan-id=200 vlan-header=always-strip

set ether10 default-vlan-id=200 vlan-header=always-strip

set ether11 default-vlan-id=200 vlan-header=always-strip

set ether12 default-vlan-id=200 vlan-header=always-strip

set ether13 default-vlan-id=200 vlan-header=always-strip

set ether14 default-vlan-id=200 vlan-header=always-strip

set ether15 default-vlan-id=200 vlan-header=always-strip

set ether16 default-vlan-id=200 vlan-header=always-strip

set ether0 vlan-header=add-if-missing

/interface ethernet

set ether17 disabled=yes

set ether18 disabled=yes

set ether19 disabled=yes

set ether20 disabled=yes

set ether21 disabled=yes

set ether22 disabled=yes

set ether23 disabled=yes

set ether24 disabled=yes

/interface bridge filter

add action=drop chain=forward in-interface=ether1 src-mac-address=!00:6B:7C:8D:9E:AF

add action=drop chain=forward in-interface=ether2 src-mac-address=!00:6C:7D:8E:9F:AA

add action=drop chain=forward in-interface=ether3 src-mac-address=!00:6D:7E:8F:9A:AB

add action=drop chain=forward in-interface=ether4 src-mac-address=!00:6E:7F:8A:9B:AC

add action=drop chain=forward in-interface=ether5 src-mac-address=!00:6F:7A:8B:9C:AD

add action=drop chain=forward in-interface=ether6 src-mac-address=!00:7A:8B:9C:AD:BE

add action=drop chain=forward in-interface=ether7 src-mac-address=!00:7B:8C:9D:AE:BF

add action=drop chain=forward in-interface=ether8 src-mac-address=!00:7C:8D:9E:AF:BA

add action=drop chain=forward in-interface=ether9 src-mac-address=!00:7D:8E:9F:AA:BB

add action=drop chain=forward in-interface=ether10 src-mac-address=!00:7E:8F:9A:AB:BC

add action=drop chain=forward in-interface=ether11 src-mac-address=!00:7F:8A:9B:AC:BD

add action=drop chain=forward in-interface=ether12 src-mac-address=!00:8A:9B:AC:BD:CE

add action=drop chain=forward in-interface=ether13 src-mac-address=!00:8B:9C:AD:BE:CF

add action=drop chain=forward in-interface=ether14 src-mac-address=!00:8C:9D:AE:BF:CA

add action=drop chain=forward in-interface=ether15 src-mac-address=!00:5D:6E:7F:8A:9B

add action=drop chain=forward in-interface=ether16 src-mac-address=!00:4F:5A:6B:7C:8D/interface vlan

add name=VLAN100 vlan-id=100 interface=bridge1

/ip address

add address=109.44.129.80/27 interface=VLAN100

/ipv6 address

add address=2001:67c:18a8:1::16 interface=VLAN10

Точка доступа AP1.1

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.2/24 interface=ether2

/ipv6 address add address=2001:67c:18a8:1::3/64 interface=ether2

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP1.2

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.3/24 interface=ether3

/ipv6 address add address=2001:67c:18a8:1::4/64 interface=ether3

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP1.3

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.4/24 interface=ether4

/ipv6 address add address=2001:67c:18a8:1::5/64 interface=ether4

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP1.4

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.5/24 interface=ether5

/ipv6 address add address=2001:67c:18a8:1::6/64 interface=ether5

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP1.5

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.6/24 interface=ether6

/ipv6 address add address=2001:67c:18a8:1::7/64 interface=ether6

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP2.1

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.7/24 interface=ether1

/ipv6 address add address=2001:67c:18a8:1::8/64 interface=ether1

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP2.2

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.1

/ip address add address=109.44.128.8/24 interface=ether2

/ipv6 address add address=2001:67c:18a8:1::9/64 interface=ether2

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP2.3

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.3

/ip address add address=109.44.128.9/24 interface=ether3

/ipv6 address add address=2001:67c:18a8:1::10/64 interface=ether1

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP2.4

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.4

/ip address add address=109.44.128.10/24 interface=ether4

/ipv6 address add address=2001:67c:18a8:1::11/64 interface=ether1

/ipv6 route add gateway=2001:67c:18a8:1::1/64

Точка доступа AP2.5

/interface wireless cap

set enabled=yes caps-man-addresses=109.44.128.5

/ip address add address=109.44.128.11/24 interface=ether5

/ipv6 address add address=2001:67c:18a8:1::12/64 interface=ether1

/ipv6 route add gateway=2001:67c:18a8:1::1/64

sudo apt update

sudo apt install samba

sudo nano /etc/network/interfaces

Файл interfaces

auto eth0

iface eth0 inet static

address 109.44.129.2

netmask 255.255.255.192

gateway 109.44.129.1

iface eth0 inet6 static

address 2001:67c:18a8:2::2

netmask 64

gateway 2001:67c:18a8:2::1

sudo systemctl restart networking

sudo groupadd samba_admins

sudo groupadd samba_users

sudo useradd user1 -s /sbin/nologin

echo "racer1626" | sudo smbpasswd -a user1

sudo usermod -aG samba_admins user1

sudo useradd user2 -s /sbin/nologin

echo "webdsg24" | sudo smbpasswd -a user2

sudo usermod -aG samba_users user2

sudo mkdir -p /srv/samba/admin_share

sudo mkdir -p /srv/samba/user_share

sudo chown :samba_admins /srv/samba/admin_share

sudo chmod 770 /srv/samba/admin_share

sudo chown :samba_users /srv/samba/user_share

sudo chmod 770 /srv/samba/user_share

sudo setfacl -m g:samba_admins:rwx /srv/samba/user_share

Файл smb.conf

[global]

workgroup = WORKGROUP

server string = Samba Server %v

netbios name = samba

security = user

map to guest = Bad User

dns proxy = no

[AdminShare]

path = /srv/samba/admin_share

available = yes

valid users = @samba_admins

read only = no

browsable = yes

public = no

writable = yes

[UserShare]

path = /srv/samba/user_share

available = yes

valid users = @samba_users, @samba_admins

read only = no

browsable = yes

public = no

writable = yes

sudo systemctl restart smbd

sudo systemctl restart nmbd