Конфигурация коммутатора SW2:

enable

configure

hostname SW2

service password-encryption

enable secret 0 sh@sji!4jFpo

username admin password 0 sh@sji!4jFpo privilege admin

ip ssh server

ip ssh version 2

line vty 0 4

login local

transport input ssh

exit

vlan 10

name ADMIN

exit

vlan 20

name EMP_WIRED

exit

vlan 30

name EMP_WIFI

exit

vlan 40

name GUEST

exit

vlan 50

name SERVERS

exit

interface vlan 10

ip address 192.168.10.3 255.255.255.240

exit

ip management-vlan 10

power inline consumption 160

power profile "AP"

supply enable

priority high

consumption class4

exit

interface 1

vlan_trunk

switchport general allowed vlan 10,20,30,40,50 tagged

exit

interface 2

switchport general allowed vlan 10 untagged

switchport general allowed vlan 30 tagged

switchport pvid 10

power inline supply enable

power inline profile "AP"

exit

interface 3

switchport general allowed vlan 10 untagged

switchport general allowed vlan 30,40 tagged

switchport pvid 10

power inline supply enable

power inline profile "AP"

exit

interface range 4,14,15

switchport general allowed vlan 50 untagged

switchport pvid 50

exit

interface range 5-13

switchport general allowed vlan 20 untagged

switchport pvid 20

exit

interface 16

shutdown

exit

end